# Technical Note ¾Using IDE Hard Drives

Batman, Batman's Revenge, Robin, Robin's Revenge, Robin LC,Plato, Socrates

## Principles of IDE Operation

·	Two types of IDE interfaces, PCI and ISA, are supported.  The interfaces, not all of which appear on all systems, are onboard PCI, onboard ISA, PCI/IDE add-in card and ISA/IDE add-in card.

·	A total of four disk drives may be used so long as they are not configured using more than two interfaces simultaneously.

·	When using only one interface, that interface is the primary interface and uses interrupt 14.

·	When using one PCI and one ISA interface, PCI becomes the primary interface using interrupt 14 and ISA becomes the secondary interface using interrupt 15.

·	When using two ISA interfaces, the onboard interface becomes the primary interface using interrupt 14 and the add-in card becomes the secondary interface using interrupt 15; the ISA add-in card must be able to be jumpered to interrupt 15.

·	When only one interface is used, interrupt 15 normally used by the secondary interface,  is available for any add-in card that can be jumpered to interrupt 15.

·	The bootup drive will always use interrupt 14.

## Usage Considerations

·	The first drive connected to any interface must be jumpered as a Master and the second as Slave.  Each interface must have one master. IDE CDROMs need to be set as slaves on the PCI IDE interface. The reason being; they speak Packet Data Transfer and not ATA. Our BIOS will look for ATA drive to be the master of the IDE channel being used, otherwise it will shut off the interface.

·	The Intel PCI/IDE add-in card can be used on Batman motherboards only and will not work with any Robin motherboard product.

·	No system will work using the Intel PCI/IDE add-in card and the ISA/IDE add-in card as the only two interfaces.  This is an exception to the rules above and exists because the BIOS forces the use of onboard ISA when it sees the Intel PCI/IDE add-in card.

·	When the Intel PCI/IDE add-in card is used, it must be inserted into PCI slot three.  No testing has been done using non-Intel PCI add-in cards.

·	Disabling  IDE using the BIOS Setup program disables both onboard ISA and onboard PCI, provided there are no drives connected to the onboard headers.

·	SCSI CD-ROMs as well as CD-ROMs with proprietary IDE interfaces may be added in addition to the 4 devices described.  They will not affect the tables following.

·	N.B. In the tables following there are references to two types of regular Robins, those with and those without PCI/IDE on the motherboard.

·	All references in the tables below to PCI/IDE add-in cards are for the Intelâ PCI/IDE add-in card; all references to ISA/IDE add-in cards are for the Boca IDEG21 add-in card.  Do not assume non-Intel PCI/IDE add-in cards work in the same configurations and the same ways as the Intel card.

·	The CMOS configuration used was Programming Option = AUTO.

Allowable IDE Configurations

|  | Number of Drives | Interface | Assigned Drive | Interrupt Used | Comments |
| --- | --- | --- | --- | --- | --- |
| One drive | 1 | Onboard PCI | C:\ | 14 | 1,3,6,7,8 |
| Configurations |  |  |  |  |  |
|  | 1 | Onboard ISA | C:\ | 14 | 1,2,3,5,6,7,8 |
|  | 1 | PCI/IDE add-in | C:\ | 14 | 1,2 |
|  | 1 | ISA/IDE add-in | C:\ | 14 | 1,2,3,5,6,7,8 |
| Two drive | 2 | Onboard PCI | C:\ & D:\ | 14 | 1,3,6,7,8 |
| Configurations |  |  |  |  |  |
|  | 2 | Onboard ISA | C:\ & D:\ | 14 | 1,2,3,5,6,7,8 |
|  | 2 | PCI/IDE add-in | C:\ & D:\ | 14 | 1,2 |
|  | 2 | ISA/IDE add-in | C:\ & D:\ | 14 | 1,2,3,5,6,7,8 |
|  | 1 | Onboard PCI | C:\ | 14 | 3,6,7,8 |
|  | 1 | Onboard ISA | D:\ | 15 |  |
|  | 1 | Onboard PCI | C:\ | 14 | 3,4,6,7,8 |
|  | 1 | ISA add-in | D:\ | 15 |  |
|  | 1 | Onboard ISA | D:\ | 15 | 2 |
|  | 1 | PCI/IDE add-in | C:\ | 14 |  |
|  | 1 | Onboard ISA | C:\ | 14 | 4,2,3,5,7,8 |
|  | 1 | ISA add-in | D:\ | 15 |  |

|  | Number of Drives | Interface | Assigned Drive | Interrupt Used | Comments |
| --- | --- | --- | --- | --- | --- |
| Three drive | 1 | Onboard PCI | C:\ | 14 | 3,6,7,8 |
| Configurations | 2 | Onboard ISA | D:\ & E:\ | 15 |  |
|  | 2 | Onboard PCI | C:\ & D:\ | 14 | 3,6,7,8 |
|  | 1 | Onboard ISA | E:\ | 15 |  |
|  | 1 | Onboard PCI | C:\ | 14 | 3,4,6,7,8 |
|  | 2 | ISA add-in | D:\ & E:\ | 15 |  |
|  | 2 | Onboard PCI | C:\ & D:\ | 14 | 3,4,6,7,8 |
|  | 1 | ISA add-in | E:\ | 15 |  |
|  | 1 | Onboard ISA | E:\ | 15 | 2 |
|  | 2 | PCI/IDE add-in | C:\ & D:\ | 14 |  |
|  | 2 | Onboard ISA | D:\ & E:\ | 15 | 2 |
|  | 1 | PCI/IDE add-in | C:\ | 14 |  |
|  | 1 | Onboard ISA | C:\ | 14 | 4,2,3,5,6,7,8 |
|  | 2 | ISA add-in | D:\ & E:\ | 15 |  |
|  | 2 | Onboard ISA | C:\ & D:\ | 14 | 4,2,3,5,6,7,8 |
|  | 1 | ISA add-in | E:\ | 15 |  |
| Four drive | 2 | Onboard PCI | C:\ & D:\ | 14 | 3,6,7,8 |
| Configurations | 2 | Onboard ISA | E:\ & F:\ | 15 |  |
|  | 2 | Onboard PCI | C:\ & D:\ | 14 | 3,4,6,7,8 |
|  | 2 | ISA add-in | E:\ & F:\ | 15 |  |
|  | 2 | Onboard ISA | E:\ & F:\ | 15 | 2 |
|  | 2 | PCI/IDE add-in | C:\ & D:\ | 14 |  |
|  | 2 | Onboard ISA | C:\ & D:\ | 14 | 4,2,3,5,6,7,8 |
|  | 2 | ISA add-in | E:\ & F:\ | 15 |  |